RigMind-NWIS — Comprehensive Project Documentation

AI-Powered Offset Well Knowledge & Decision Support Platform for Oil India Limited (OIL)

1. Executive Overview & Problem Statement

RigMind-NWIS is an industrial decision-support platform designed for field drilling engineers and superintendents operating in oilfields (specifically designed for Oil India Limited - SIH 2026).

In upstream oil and gas operations, non-productive time (NPT) caused by hazardous drilling conditions—such as stuck pipes, severe mud circulation losses, and unexpected formation overpressure (kicks)—costs petroleum operators millions of dollars per well.

RigMind-NWIS addresses this by correlating real-time drilling telemetry with historical offset well reports within a 5 km – 10 km geographic radius. By incorporating geological dip corrections and RAG (Retrieval-Augmented Generation) historical report evidence, RigMind-NWIS predicts hazards before they occur and presents verified mitigation steps directly from past completion and daily drilling reports.

2. System Architecture & Implementation Details

The project is engineered as a decoupled, multi-tiered microservice architecture designed for resilience and sub-second operational response times:

| Component | Technologies & Frameworks | Core Implementation Role |
| --- | --- | --- |
| Frontend UI Layer | React 18, Vite, Tailwind CSS, Plotly 3D, Chart.js, React-Leaflet GIS, Socket.IO Client | Single-Page Application providing 2D GIS map, interactive 3D subsurface trajectory model, 3-column live drilling dashboard, and administrative PDF vector upload interface. |
| Core Backend Server | Node.js (v18+), Express.js, JWT Auth, PostGIS Client (`pg`), Socket.IO Server | Manages user authentication, role-based authorization, REST API routing, PostGIS spatial queries (`ST_DWithin`), real-time WebSocket telemetry engine, and proxy router to AI Service. |
| AI & RAG Service Engine | Python 3.11, FastAPI, SciPy, ChromaDB, LangChain, Groq API (with Gemini fallback) | Executes 3D Stratigraphic Dip Correction math, processes deterministic multi-parameter risk scores, manages vector chunking, similarity search in ChromaDB, and LLM synthesis. |
| Relational & Spatial Data Store | PostgreSQL 15 + PostGIS 3.3 (SRID 4326) | Stores user credentials, master well directory with GIS geometry points, trajectory logs, formation stratigraphy, historical drilling incident logs, and report extraction metadata. |

3. Operational Workflow (How It Works)

The operation of RigMind-NWIS follows a 6-step lifecycle:

Step 1: Role Authentication — Users log in via JWT authentication. The system grants permissions based on the role assigned (Field Engineer vs. Data Admin).

Step 2: Well Initialization & Spatial Search — The engineer inputs target well coordinates (Latitude: 27.38°N, Longitude: 95.32°E), desired search radius (10 km), and target depth (3500m). The backend executes a PostGIS ST_DWithin query to identify nearby historical wells (e.g. NHK-101, NHK-112, NHK-104, DGB-88).

Step 3: 3D Trajectory & Geological Dip Correlation — The 3D subsurface viewer renders the planned trajectory alongside offset well paths. Activating 'Stratigraphic Dip Correction' recalculates formation top depths using regional structural dip angle (tan θ offset shift).

Step 4: Live Telemetry Streaming — Clicking 'START DRILLING' initiates the Socket.IO telemetry streamer. Real-time drilling telemetry (Depth, ROP, WOB, RPM, Torque, SPP, Flow Rate, Mud Weight) updates every 2 seconds.

Step 5: Dynamic Risk Detection — As the bit depth passes 2820m in the Barail Main Formation, parameter anomalies (RPM dropping below 70 RPM, Torque spiking above 25 kN.m) trigger a HIGH RISK — STUCK PIPE alert (Risk Score 0.88).

Step 6: RAG Historical Investigation — The user clicks 'Investigate Historical Solutions'. RigMind-NWIS queries ChromaDB vector store for historical reports from offset wells NHK-101 and NHK-112, retrieving exact historical mitigations (spotting 50 bbl high-lubricity pill, boosting flow rate to 520 gpm, downward jarring) with source document citations.

4. Comprehensive Parameter Dictionary

RigMind-NWIS relies on three categories of operational parameters:

4.1 Real-Time Telemetry Parameters

• Measured Depth (MD - m): Current vertical depth reached by the drill bit.

• Rate of Penetration (ROP - m/hr): Speed at which the drill bit deepens the borehole.

• Weight on Bit (WOB - klbs): Downward force applied on the drill bit.

• Rotary Speed (RPM): Rotational speed of the drillstring.

• Torque (kN.m / ft-lbs): Rotational resistance experienced by the drillstring. Torque spikes are primary indicators of pipe sticking.

• Standpipe Pressure (SPP - psi): Pressure of drilling fluid pumped downhole. Drops indicate mud loss or washouts; spikes indicate flow line restriction.

• Flow Rate (gpm): Volume of drilling fluid circulated per minute. Critical for cuttings removal.

• Mud Weight (sg / ppg): Specific gravity / density of drilling fluid used to maintain wellbore hydrostatic balance.

4.2 Spatial & Geological Parameters

• Latitude / Longitude (SRID 4326): WGS 84 spatial point geometry used by PostGIS for distance calculation.

• Survey Radius (km): Geographic search boundary (e.g. 5 km or 10 km) for selecting offset wells.

• Dip Angle (θ - degrees): Regional planar tilt of rock strata (e.g. 5.0° SE in Assam basin).

• Structural Depth Shift (m): Depth adjustment delta computed as: Depth Shift = Distance (m) * tan(Dip Angle).

4.3 Risk Evaluation Threshold Parameters

• Stuck Pipe Rules: Depth Proximity <= 100m AND (RPM < 70 OR Torque > 25 kN.m OR WOB > 50 klbs). Score threshold >= 0.65.

• Mud Loss Rules: Depth Proximity <= 80m AND (Flow Rate drop > 15% OR SPP drop > 200 psi).

• Kick / Overpressure Rules: Depth Proximity <= 100m AND (SPP drop > 150 psi AND ROP spike > 40%).

5. Technology Stack Summary

Below is the complete technology stack powering RigMind-NWIS:

| Category | Technologies & Tools |
| --- | --- |
| Frontend | React 18, Vite, Tailwind CSS, Lucide React Icons, React Router v6 |
| 3D & 2D Data Visualization | Plotly.js (react-plotly.js), Chart.js (react-chartjs-2), Leaflet / React-Leaflet |
| Backend Services | Node.js, Express.js, jsonwebtoken (JWT), Socket.IO (WebSockets), CORS |
| AI / ML & Geoscience Engine | Python 3.11, FastAPI, Uvicorn, SciPy, NumPy, PyPDF |
| RAG & Vector Store | ChromaDB, LangChain, Groq API (LLaMA-3.1-8b-instant), Google Gemini API Fallback |
| Database & GIS Storage | PostgreSQL 15, PostGIS 3.3 (Spatial indexing with GiST, SRID 4326 point geometries) |

6. Role-Based Routing & Access Control (RBAC)

RigMind-NWIS enforces strict security and role separation across both the frontend React UI and backend Express routes:

6.1 Client-Side Protected Routing (`App.jsx`)

The application utilizes a `ProtectedRoute` wrapper component that evaluates JWT token validity and role authorization prior to rendering any layout component:

• `/login` — Public login page where users authenticate as either `FIELD_ENGINEER` or `DATA_ADMIN`.

• `/initialize` — Accessible by both roles. Allows setting target well coordinates, target depth, and triggering PostGIS spatial offset well selection.

• `/subsurface-map` — Accessible by both roles. Renders 3D Plotly subsurface trajectory model with dip correction toggle.

• `/dashboard` — Accessible by both roles. Provides 3-column real-time telemetry streaming, chart visualizations, risk engine alerts, and RAG investigation modal.

• `/knowledge-base` — Accessible by both roles. Searchable interface for browsing historical offset well event records.

• `/admin` — RESTRICTED ROUTE (DATA_ADMIN only). Allows uploading new Daily Drilling Reports (DDR) / Well Completion Reports (WCR) in PDF format and triggering ChromaDB vector indexing.

6.2 Backend Permission Enforcement

The Node backend enforces token verification via `verifyToken` middleware and role checks via `requireRole(['DATA_ADMIN'])`. Unauthenticated requests return HTTP 401 Unauthorized, while unauthorized role attempts return HTTP 403 Forbidden.

7. Graphs and Visualizations Specification

Data visualization is central to RigMind-NWIS. Every graph in the application serves a specific engineering objective:

7.x 3D Subsurface Trajectory & Stratigraphy Map (Plotly 3D)

Page Location: SubsurfaceMapPage.jsx

Visual Representation: Displays target well trajectory in solid bright green, nearby historical offset well trajectories in dashed grey, and formation top boundaries as 3D surface meshes.

Engineering Rationale (Why it is used): Allows drilling superintendents to visually assess wellbore proximity relative to historical offset wells and identify potential collision risks or fault planes in 3D space.

7.x 2D Interactive Spatial GIS Map (Leaflet)

Page Location: InitializePage.jsx

Visual Representation: Displays geographic latitude/longitude wellhead positions in the Upper Assam basin, radial search boundary circles (5km / 10km), and color-coded historical hazard pins.

Engineering Rationale (Why it is used): Provides geographical orientation and spatial context in standard map projection (SRID 4326).

7.x Real-Time Telemetry Line Charts (Chart.js)

Page Location: DashboardPage.jsx

Visual Representation: 4 synchronized time-series graphs streaming bit depth (m), RPM, Torque (kN.m), and Weight on Bit (klbs).

Engineering Rationale (Why it is used): Enables rig crew to monitor parameter trends. Sharp divergence in parameters (e.g. Torque rising while RPM drops) signals immediate physical hazard onset.

7.x Vertical Depth Timeline / Risk Barcode

Page Location: DashboardPage.jsx

Visual Representation: A vertical depth progress gauge representing 0m to 3500m depth, overlaid with color-coded formation depth bands and historical hazard tags.

Engineering Rationale (Why it is used): Gives engineers a vertical preview of upcoming high-risk geological strata before the bit reaches those depths.

7.x Dynamic Risk Gauge & Alert Banner

Page Location: DashboardPage.jsx

Visual Representation: Dynamic risk score meter (0.0 to 1.0) with hazard status badge (GREEN / YELLOW / RED) and action buttons.

Engineering Rationale (Why it is used): Acts as an un-missable operational warning signal that automatically prompts the user to launch RAG evidence investigation.